CGT50 Pre Flight Checklist / CGT50 飛行前檢查表

VERSION / 版本: V 18.0

EFFECTIVE DATE / 生效日期: 3 April 2023 / 2023年4月3日

Read & Do / 閱讀並執行

[ ] = If Applicable / 若適用

| 1. DETAILS / 詳細資訊 |  |  |  |
| --- | --- | --- | --- |
| 1 | Tail Number: 機尾編號: |  |  |
| 2 | Flight Location: 飛行地點: |  |  |
| 3 | Date: 日期: |  |  |
| 4 | Time: 時間: |  |  |
| 5 | Pre Flight Checklist Completed By: 技術人員姓名 (Technician Name): 飛行前檢查表完成者: |  |  |

| 2. FLIGHTLINE FOD CHECKS (TAKEOFF & LANDING AREAS) / 停機坪外來物 (FOD) 檢查 (起飛與降落區域) |  |  |  |
| --- | --- | --- | --- |
| 1 | Nearby Perimeter Fences 附近周邊圍欄 | Clean 乾淨 | [ ] |
| 2 | Outside Rubbish Bins 外部垃圾桶 | Covered, Not Overflowing 有蓋，未滿溢 | [ ] |
| 3 | Construction Zones 施工區域 | Clean or Protected by Fencing 乾淨或有圍欄保護 | [ ] |
| 4 | Loose Objects in the Flight Vicinity 飛行附近鬆動物體 | None 無 | [ ] |
| 5 | Access Areas to the Aircraft 飛機通道區域 | Free of Rubbish 無垃圾 | [ ] |
| 6 | Hardware or Other Debris on Ramp Surface 停機坪表面的五金件或其他碎片 | None 無 | [ ] |
| 7 | Taxiway Shoulders 滑行道道肩 | Free of Loose Stones & Grass Cuttings 無鬆動石塊與割下的碎草 | [ ] |
| 8 | Infields 內場 (草坪區) | Free of Rubbish 無垃圾 | [ ] |
| 9 | Objects that may be Dangerously Close to the Launching Aircraft 可能危險靠近起飛飛機的物體 | None 無 | [ ] |
| 10 | Maintenance Personnel 維修人員 | Neat, FOD Free, Nothing can Come Loose 整潔，無FOD，無鬆動物品 | [ ] |
| 11 | Bird & Wildlife Attractants Nearby 附近有吸引鳥類與野生動物的物品 | Check 檢查 | [ ] |
| 12 | Large Birds Visible from the Airfield 從機場可見的大型鳥類 | Check 檢查 | [ ] |
| 13 | FOD Containers Accessible to Work Areas 工作區域可使用的FOD收集容器 | Confirm 確認 | [ ] |
| 14 | TakeOff Area 起飛區域 | Clear of Any Debris 無任何碎片 | [ ] |
| 15 | Landing & Alternate Landing Areas 降落與備用降落區域 | Clear of Any Debris 無任何碎片 | [ ] |

| 3. GENERAL / 一般事項 |  |  |  |
| --- | --- | --- | --- |
| 1 | Personal Protective Equipment (PPE) UAS Team 個人防護裝備 (PPE) 無人機團隊 | Checked 已檢查 | [ ] |
| 2 | Minimum Technical Equipment (Required for all Operating Procedures) 最低技術裝備 (所有操作程序所需) | Available 可用 | [ ] |
| 3 | Safety Equipment (Required for all Operating Procedures) 安全裝備 (所有操作程序所需) | Available, Checked 可用，已檢查 | [ ] |
| 4 | Move Aircraft from Assembly Area to TakeOff Area 將飛機從組裝區移至起飛區 | Min 2 People 至少2人 | [ ] |
| 5 | TakeOff Table 起飛台 | Assembled, Secure 已組裝，穩固 | [ ] |
| 6 | TakeOff Table [ ] (Consider Engine Start & RunUp, 2 Crew Operations) 起飛台 [ ] (考慮引擎啟動與試車，2名機組員操作) | Secured to Ground 固定於地面 | [ ] |
| 7 | Aircraft Positioned on TakeOff Table 飛機放置於起飛台上 | Check Table Markings 檢查平台標記 | [ ] |
| 8 | Aircraft & Table 飛機與平台 | Orientated Into Wind 面向迎風面 | [ ] |
| 9 | GCS Ignition [ ] GCS 點火開關 [ ] | Off 關閉 | [ ] |
| 10 | Aircraft Ignition 飛機點火開關 | Off 關閉 | [ ] |
| 11 | LH Pitot Tube 左側 (LH) 皮托管 | Undamaged 無損壞 | [ ] |
| 12 | LH Static Port 左側 (LH) 靜壓孔 | Undamaged 無損壞 | [ ] |
| 13 | RH Pitot Tube 右側 (RH) 皮托管 | Undamaged 無損壞 | [ ] |
| 14 | RH Static Port 右側 (RH) 靜壓孔 | Undamaged 無損壞 | [ ] |
| 15 | All Covers (Pitot, Propeller, Exhaust) 所有護罩 (皮托管、螺旋槳、排氣管) | Installed 已安裝 | [ ] |
| 16 | Undercarriage & Tail Rest 起落架與機尾支架 | Undamaged, Secure 無損壞，穩固 | [ ] |
| 17 | Fuselage (Lower, LHS, Upper, RHS) 機身 (下部、左側、上部、右側) | Undamaged 無損壞 | [ ] |
| 18 | All Antennas 所有天線 | Secured, Undamaged 穩固，無損壞 | [ ] |
| 19 | XT60 Regulator Connection XT60 穩壓器連接 | Connected 已連接 | [ ] |
| 20 | Shore Power (Adjust to 14.5 V) 岸電 (調整至 14.5 V) | Connect 連接 | [ ] |
| 21 | Service Covers & Screws (0.3 Nm Torque) 維修蓋板與螺絲 (0.3 Nm 扭力) | Installed, Secure 已安裝，穩固 | [ ] |

| 4. LH (PORT) WING / 左側機翼 |  |  |  |
| --- | --- | --- | --- |
| 1 | Wing (Leading Edge, Upper, Trailing Edge, Lower) 機翼 (前緣、上部、後緣、下部) | Undamaged 無損壞 | [ ] |
| 2 | Ailerons (Both) (Upper, Trailing Edge, Lower) 副翼 (兩側) (上部、後緣、下部) | Undamaged 無損壞 | [ ] |
| 3 | Wing Tip LED 翼尖 LED 燈 | Undamaged 無損壞 | [ ] |
| 4 | Servos 伺服馬達 | Undamaged 無損壞 | [ ] |
| 5 | Servo Covers (Covers 0.3 Nm Torque) 伺服馬達護蓋 (護蓋 0.3 Nm 扭力) | Secure 穩固 | [ ] |
| 6 | Servo Push Rods 伺服推桿 | Undamaged, Not Bent 無損壞，無彎曲 | [ ] |
| 7 | Servo Arms 伺服擺臂 | Undamaged 無損壞 | [ ] |
| 8 | Servo Arms 伺服擺臂 | No Play Present 無間隙 | [ ] |
| 9 | Control Surface Horns 控制面角 | Undamaged, Secure 無損壞，穩固 | [ ] |
| 10 | Pushrod Clips 推桿夾 | In Place, Secure 就位，穩固 | [ ] |
| 11 | Control Surfaces & Hinges 控制面與鉸鏈 | Operate Smoothly, No Wear or Damage 運作順暢，無磨損或損壞 | [ ] |

| 5. LH (PORT) VTOL SYSTEM / 左側垂直起降系統 |  |  |  |
| --- | --- | --- | --- |
| 1 | VTOL Boom (Longer Section to the Front, Fasteners 5 Nm Torque) VTOL 尾樑 (較長部分朝前，緊固件 5 Nm 扭力) | Attached, Secure 已連接，穩固 | [ ] |
| 2 | VTOL Motor Assemblies (Both) VTOL 馬達組件 (兩側) | Secure & No Loose or Missing Screws 穩固且無鬆動或遺失的螺絲 | [ ] |
| 3 | VTOL Motor Housing & Windings (Both) (Dry Air Only to be Used if Blown Clean) VTOL 馬達外殼與繞組 (兩側) (若需吹乾淨只能使用乾燥空氣) | Clean, No Dirt, Debris, FOD 乾淨，無污垢、碎片、FOD | [ ] |
| 4 | VTOL Motor Cable Sheaths (Both) VTOL 馬達電纜護套 (兩側) | Undamaged, Unfrayed, No Cuts or Pinches 無損壞、無磨損、無割傷或擠壓 | [ ] |
| 5 | VTOL ESC Cables VTOL ESC 電纜 | Undamaged, Unfrayed, No Cuts or Pinches 無損壞、無磨損、無割傷或擠壓 | [ ] |
| 6 | Vtol ESC Mount Plate VTOL ESC 安裝板 | Bonded, Undamaged 黏合良好，無損壞 | [ ] |
| 7 | Propeller Blades (Both Motor Assemblies) 螺旋槳葉片 (兩側馬達組件) | Undamaged, Secure 無損壞，穩固 | [ ] |
| 8 | Propellers FWD Motor 前置馬達螺旋槳 | Labelled L or CW 標示 L 或 CW | [ ] |
| 9 | Propellers AFT Motor 後置馬達螺旋槳 | Labelled R or CCW 標示 R 或 CCW | [ ] |
| 10 | Both VTOL Motors Tilted Outwards 兩側 VTOL 馬達皆向外傾斜 | Confirm 確認 | [ ] |

| 6. VERTICAL STABILISER / 垂直尾翼 |  |  |  |
| --- | --- | --- | --- |
| 1 | Vertical Stabiliser (Leading Edge, LHS, Trailing Edge, RHS) 垂直尾翼 (前緣、左側、後緣、右側) | Undamaged 無損壞 | [ ] |
| 2 | Rudder (LHS, Trailing Edge, RHS) 方向舵 (左側、後緣、右側) | Undamaged 無損壞 | [ ] |
| 3 | Servo 伺服馬達 | Undamaged 無損壞 | [ ] |
| 4 | Servo Cover (Covers 0.3 Nm Torque) 伺服馬達護蓋 (護蓋 0.3 Nm 扭力) | Secure 穩固 | [ ] |
| 5 | Servo Push Rod 伺服推桿 | Undamaged, Not Bent 無損壞，無彎曲 | [ ] |
| 6 | Servo Arm 伺服擺臂 | Undamaged 無損壞 | [ ] |
| 7 | Servo Arm 伺服擺臂 | No Play Present 無間隙 | [ ] |
| 8 | Control Surface Horn 控制面角 | Undamaged, Secure 無損壞，穩固 | [ ] |
| 9 | Pushrod Clips 推桿夾 | In Place, Secure 就位，穩固 | [ ] |
| 10 | Control Surfaces & Hinges 控制面與鉸鏈 | Operate Smoothly, No Wear or Damage 運作順暢，無磨損或損壞 | [ ] |

| 7. HORIZONTAL STABILISER / 水平尾翼 |  |  |  |
| --- | --- | --- | --- |
| 1 | Horizontal Stabiliser (Leading Edge, Upper, Trailing Edge, Lower) 水平尾翼 (前緣、上部、後緣、下部) | Undamaged 無損壞 | [ ] |
| 2 | LH Elevator (Upper, Trailing Edge, Lower) 左側升降舵 (上部、後緣、下部) | Undamaged 無損壞 | [ ] |
| 3 | RH Elevator (Upper, Trailing Edge, Lower) 右側升降舵 (上部、後緣、下部) | Undamaged 無損壞 | [ ] |
| 4 | Servos 伺服馬達 | Undamaged 無損壞 | [ ] |
| 5 | Servo Covers (Covers 0.3 Nm Torque) 伺服馬達護蓋 (護蓋 0.3 Nm 扭力) | Secure 穩固 | [ ] |
| 6 | Servo Push Rods 伺服推桿 | Undamaged, Not Bent 無損壞，無彎曲 | [ ] |
| 7 | Servo Arms 伺服擺臂 | Undamaged 無損壞 | [ ] |
| 8 | Servo Arms 伺服擺臂 | No Play Present 無間隙 | [ ] |
| 9 | Control Surface Horns 控制面角 | Undamaged, Secure 無損壞，穩固 | [ ] |
| 10 | Pushrod Clips 推桿夾 | In Place, Secure 就位，穩固 | [ ] |
| 11 | Control Surfaces & Hinges 控制面與鉸鏈 | Operate Smoothly, No Wear or Damage 運作順暢，無磨損或損壞 | [ ] |

| 8. PILOT CAMERA / 飛行員攝影機 |  |  |  |
| --- | --- | --- | --- |
| 1 | Support & Cover 支架與護蓋 | Undamaged, Secure 無損壞，穩固 | [ ] |
| 2 | Camera 攝影機 | Aligned Correctly 對齊正確 | [ ] |
| 3 | Camera Lens 攝影機鏡頭 | Undamaged 無損壞 | [ ] |
| 4 | Camera Lens 攝影機鏡頭 | Clean 乾淨 | [ ] |
| 5 | Camera Lens 攝影機鏡頭 | No Fog, Moisture or Residue 無起霧、濕氣或殘留物 | [ ] |

| 9. RH (STARBOARD) WING / 右側機翼 |  |  |  |
| --- | --- | --- | --- |
| 1 | Wing (Leading Edge, Upper, Trailing Edge, Lower) 機翼 (前緣、上部、後緣、下部) | Undamaged 無損壞 | [ ] |
| 2 | Ailerons (Both) (Upper, Trailing Edge, Lower) 副翼 (兩側) (上部、後緣、下部) | Undamaged 無損壞 | [ ] |
| 3 | Wing Tip LED 翼尖 LED 燈 | Undamaged 無損壞 | [ ] |
| 4 | Servos 伺服馬達 | Undamaged 無損壞 | [ ] |
| 5 | Servo Covers (Covers 0.3 Nm Torque) 伺服馬達護蓋 (護蓋 0.3 Nm 扭力) | Secure 穩固 | [ ] |
| 6 | Servo Push Rods 伺服推桿 | Undamaged, Not Bent 無損壞，無彎曲 | [ ] |
| 7 | Servo Arms 伺服擺臂 | Undamaged 無損壞 | [ ] |
| 8 | Servo Arms 伺服擺臂 | No Play Present 無間隙 | [ ] |
| 9 | Control Surface Horns 控制面角 | Undamaged, Secure 無損壞，穩固 | [ ] |
| 10 | Pushrod Clips 推桿夾 | In Place, Secure 就位，穩固 | [ ] |
| 11 | Control Surfaces & Hinges 控制面與鉸鏈 | Operate Smoothly, No Wear or Damage 運作順暢，無磨損或損壞 | [ ] |

| 10. RH (STARBOARD) VTOL SYSTEM / 右側垂直起降系統 |  |  |  |
| --- | --- | --- | --- |
| 1 | VTOL Boom (Longer Section to the Front, Fasteners 5 Nm Torque) VTOL 尾樑 (較長部分朝前，緊固件 5 Nm 扭力) | Attached, Secure 已連接，穩固 | [ ] |
| 2 | VTOL Motor Assemblies (Both) VTOL 馬達組件 (兩側) | Secure & No Loose or Missing Screws 穩固且無鬆動 or 遺失的螺絲 | [ ] |
| 3 | VTOL Motor Housing & Windings (Both) (Dry Air Only to be Used if Blown Clean) VTOL 馬達外殼與繞組 (兩側) (若需吹乾淨只能使用乾燥空氣) | Clean, No Dirt, Debris, FOD 乾淨，無污垢、碎片、FOD | [ ] |
| 4 | VTOL Motor Cable Sheaths (Both) VTOL 馬達電纜護套 (兩側) | Undamaged, Unfrayed, No Cuts or Pinches 無損壞、無磨損、無割傷或擠壓 | [ ] |
| 5 | VTOL ESC Cables VTOL ESC 電纜 | Undamaged, Unfrayed, No Cuts or Pinches 無損壞、無磨損、無割傷或擠壓 | [ ] |
| 6 | Vtol ESC Mount Plate VTOL ESC 安裝板 | Bonded, Undamaged 黏合良好，無損壞 | [ ] |
| 7 | Propeller Blades (Both Motor Assemblies) 螺旋槳葉片 (兩側馬達組件) | Undamaged, Secure 無損壞，穩固 | [ ] |
| 8 | Propellers FWD Motor 前置馬達螺旋槳 | Labelled R or CCW 標示 R 或 CCW | [ ] |
| 9 | Propellers AFT Motor 後置馬達螺旋槳 | Labelled L or CW 標示 L 或 CW | [ ] |
| 10 | Both VTOL Motors Tilted Outwards 兩側 VTOL 馬達皆向外傾斜 | Confirm 確認 | [ ] |

| 11. MAIN ENGINE / 主引擎 |  |  |  |
| --- | --- | --- | --- |
| 1 | GCS Ignition [ ] GCS 點火開關 [ ] | Off 關閉 | [ ] |
| 2 | Aircraft Ignition 飛機點火開關 | Off 關閉 | [ ] |
| 3 | Cowling [ ] 引擎整流罩 [ ] | Undamaged, Secure 無損壞，穩固 | [ ] |
| 4 | Engine (May be Hot) 引擎 (可能高溫) | Undamaged 無損壞 | [ ] |
| 5 | Exhaust (Mufflers) 排氣管 (消音器) | Undamaged, Debris Free 無損壞，無碎片 | [ ] |
| 6 | All Torque Seals 所有扭力標記 | Aligned 對齊 | [ ] |
| 7 | Propeller & Blade 螺旋槳與葉片 | Secure, Undamaged 穩固，無損壞 | [ ] |
| 8 | Propeller 螺旋槳 | Rotates by Hand 可用手轉動 | [ ] |
| 9 | Propeller Stall Position (Flick X 7 from TDC) 螺旋槳失速位置 (從上死點 TDC 撥動7次) | Horizontal 水平 | [ ] |
| 10 | Propeller Attachment Bolts Torque Seals (Remove Spinner to Check, Bolts 9.5 Nm Torque) 螺旋槳連接螺栓扭力標記 (拆下整流罩檢查，螺栓 9.5 Nm 扭力) | Aligned 對齊 | [ ] |
| 11 | Spinner 螺旋槳整流罩 | Aligned, Undamaged 對齊，無損壞 | [ ] |
| 12 | Spinner Connection Screw 整流罩連接螺絲 | Tight, No Play 鎖緊，無間隙 | [ ] |
| 13 | Standoff, Vibration Mounts 支柱，減震墊 | Undamaged, Not Hard, No Cracks 無損壞，未硬化，無裂縫 | [ ] |
| 14 | All Torque Seals 所有扭力標記 | Aligned 對齊 | [ ] |
| 15 | Fuel Hose 燃油管 | Undamaged, No Leaks 無損壞，無洩漏 | [ ] |
| 16 | Spark Plugs & Cables 火星塞與電纜 | Undamaged, Not Loose, Properly Seated 無損壞，未鬆動，正確就位 | [ ] |
| 17 | Hall Sensor & Wire Harness (Check Sensor Position has Not Shifted from Reference Mark) 霍爾感測器與線束 (檢查感測器位置未偏離參考標記) | Undamaged 無損壞 | [ ] |
| 18 | Temperature Sensor 溫度感測器 | Undamaged, In Place, Secure 無損壞，就位，穩固 | [ ] |

| 12. AIRCRAFT CENTRE OF GRAVITY (CG) / 飛機重心 |  |  |  |
| --- | --- | --- | --- |
| 1 | VTOL Batteries VTOL 電池 | OnBoard, Disconnected 機上，未連接 | [ ] |
| 2 | Ballast Required (CG Adjusted by Adding or Removing Ballast at Designated Locations) 需配重 (透過在指定位置增加或移除配重來調整重心) | Added or Removed 已增加或移除 | [ ] |
| 3 | Payload [ ] 酬載 [ ] | OnBoard 機上 | [ ] |
| 4 | All Protection Covers (Pitot, Propeller, Exhaust) 所有防護罩 (皮托管、螺旋槳、排氣管) | Removed 已移除 | [ ] |
| 5 | All Service Panels & Covers 所有維修面板與護蓋 | In Place 就位 | [ ] |
| 6 | Centre of Gravity (Appropriate Method Should be Used, Fuelling Does Not Change CG Location) 重心 (應使用適當方法，加油不會改變重心位置) | Tested & True 測試並確認無誤 | [ ] |

| 13. AIRCRAFT ZERO FUEL WEIGHT / 飛機零燃油重量 |  |  |  |
| --- | --- | --- | --- |
| 1 | VTOL Batteries VTOL 電池 | OnBoard, Disconnected 機上，未連接 | [ ] |
| 2 | Payload [ ] 酬載 [ ] | OnBoard 機上 | [ ] |
| 3 | All Protection Covers (Pitot, Propeller, Exhaust) 所有防護罩 (皮托管、螺旋槳、排氣管) | Removed 已移除 | [ ] |
| 4 | All Service Panels & Covers 所有維修面板與護蓋 | In Place 就位 | [ ] |
| 5 | Record Aircraft Zero Fuel Weight 記錄飛機零燃油重量 | [ ] kg | [ ] |

| 14. FUELLING / 加油 |  |  |  |
| --- | --- | --- | --- |
| 1 | Fire Extinguisher at Fuelling Area 加油區的滅火器 | Present & Reachable 存在且可及 | [ ] |
| 2 | Fuelling Adapter 加油轉接頭 | Connect 連接 | [ ] |
| 3 | Fuelling Procedure 加油程序 | Complete 完成 | [ ] |
| 4 | Fuel Tank (Wait a Few Minutes and Check the Outside of the Fuel Tank) 油箱 (等待幾分鐘並檢查油箱外部) | No Leaks 無洩漏 | [ ] |
| 5 | Record Total Fuel 記錄總燃油量 | [ ] kg | [ ] |

| 15. AIRCRAFT TAKEOFF WEIGHT / 飛機起飛重量 |  |  |  |
| --- | --- | --- | --- |
| 1 | VTOL Batteries VTOL 電池 | OnBoard, Disconnected 機上，未連接 | [ ] |
| 2 | Payload [ ] 酬載 [ ] | OnBoard 機上 | [ ] |
| 3 | Fuelling 加油 | Completed 已完成 | [ ] |
| 4 | All Protection Covers (Pitot, Propeller, Exhaust) 所有防護罩 (皮托管、螺旋槳、排氣管) | Removed 已移除 | [ ] |
| 5 | All Service Panels & Covers 所有維修面板與護蓋 | In Place 就位 | [ ] |
| 6 | Record Aircraft TakeOff Weight (Aircraft Total Weight KDE VTOL <= 55 kg, Prototype / Motor VTOL <= 60 kg) 記錄飛機起飛重量 (飛機總重 KDE VTOL <= 55 kg, 原型機 / 馬達 VTOL <= 60 kg) | [ ] kg | [ ] |
| 7 | Pitot Covers 皮托管防護罩 | Install 安裝 | [ ] |

| 16. VTOL BATTERIES / VTOL 電池 |  |  |  |
| --- | --- | --- | --- |
| 1 | Record VTOL Battery 1 - LH Wing (% Capacity & Serial Number) 記錄 VTOL 電池 1 - 左翼 (容量百分比與序號) | [ ] % [ ] # |  |
| 2 | Record VTOL Battery 2 - LH Wing (% Capacity & Serial Number) 記錄 VTOL 電池 2 - 左翼 (容量百分比與序號) | [ ] % [ ] # |  |
| 3 | Record VTOL Battery 1 - RH Wing (% Capacity & Serial Number) 記錄 VTOL 電池 1 - 右翼 (容量百分比與序號) | [ ] % [ ] % [ ] # |  |
| 4 | Record VTOL Battery 2 - RH Wing (% Capacity & Serial Number) 記錄 VTOL 電池 2 - 右翼 (容量百分比與序號) | [ ] % [ ] # |  |
| 5 | All Cells of All VTOL Batteries 所有 VTOL 電池的所有電芯 | >= 97% Capacity >= 97% 容量 | [ ] |

------------------------------------------------ END / 結束 ------------------------------------------------