YT Delta Wing Pre Flight Checklist / 御天三角翼飛行前檢查表

VERSION / 版本: V 0.01

EFFECTIVE DATE / 生效日期: 1 SEP 2026 / 2026年9月1日

Read & Do / 逐項閱讀並執行

[ ] = If Applicable / 適用(合格打勾)

Check clockwise from left to right & from back to front/順時針檢查由左至右由後至前

| 1. DETAILS / 詳細資訊 |  |  |  |
| --- | --- | --- | --- |
| 1 | 機尾編號/ Tail Number: |  |  |
| 2 | 飛行地點(+經緯度)/light Location: |  |  |
| 3 | 場域大小(飛行場範圍)/ Field size: |  |  |
| 4 | 使用空域(+經緯度) /airspace used: |  |  |
| 5 | 起飛方式(跑道、拋飛、彈射架)/ takeoff method(runway、throw、catapult): |  |  |
| 6 | 天氣預報/ Weather forecast(幾度風幾浬) |  |  |
| 7 | 跑道方向/ runway direction:(風向及地點決定起飛方向) |  |  |
| 8 | 落地方式(跑道、攔截網、降落傘)/Landing method(Runway、interception net、Parachute): |  |  |
| 9 | Date: 日期: |  |  |
| 10 | Time: 時間: |  |  |
| 11 | 執行人數:建議2-3人(視機種調整) |  |  |
| 12 | 飛行前檢查員/Pre-Flight inspector: 飛手姓名/controller Name 技術人員/Technician:技術人員姓名/ Technician Name | 最終檢查完成者簽名/ Signature of the person who completed the final check: |  |

| 2. FLIGHTLINE FOD CHECKS (TAKEOFF & LANDING AREAS) / 飛行線FOD檢查清單 (起飛與降落區域) |  |  |  |
| --- | --- | --- | --- |
| 1 | 無事人員FOD /Idle crew members FOD | Thorough Cleaning/徹底清潔 | [ ] |
| 2 | Nearby Perimeter Fences 附近周邊圍欄 | Clean 乾淨無雜物 | [ ] |
| 3 | Outside Rubbish Bins 室外垃圾桶 | Covered, Not Overflowing 有加蓋且未溢出 | [ ] |
| 4 | Construction Zones 施工區域 | Clean or Protected by Fencing 乾淨有圍欄防護 | [ ] |
| 5 | Loose Objects in the Flight Vicinity 飛行鄰近區附近鬆動物品 | None 無 | [ ] |
| 6 | Access Areas to the Aircraft 飛機通道區域 | Free of Rubbish 無垃圾 | [ ] |
| 7 | Hardware or Other Debris on Ramp Surface 停機坪表面的五金件(硬件)或其他碎片 | None 無 | [ ] |
| 8 | Taxiway Shoulders 滑行道道肩 | Free of Loose Stones & Grass Cuttings 無鬆動石塊與割下的碎草 | [ ] |
| 9 | Infields 內場 (草坪區) | Free of Rubbish 無垃圾 | [ ] |
| 10 | Runway area/跑道區域 | Free of Loose Stones & Grass Cuttings 無鬆動石塊與割下的碎草 | [ ] |
| 11 | Objects that may be Dangerously Close to the Launching Aircraft 可能危險靠近起飛飛機的物體 | None 無 | [ ] |
| 12 | Maintenance Personnel 機組人員 | Neat, FOD Free, Nothing can Come Loose 服裝整潔，無FOD，無鬆動物品 | [ ] |
| 13 | Bird & Wildlife Attractants Nearby 附近吸引鳥類(大型鳥類)與野生動物的物品 | Check 已檢查 | [ ] |
| 15 | FOD Containers Accessible to Work Areas 工作區域可使用的FOD收集容器 | Confirm 已確認 | [ ] |
| 16 | TakeOff Area And Landing Area 起飛區域及降落區域 | Clear of Any Debris 無任何碎片 | [ ] |

| 3. GENERAL / 一般常規檢查 |  |  |  |
| --- | --- | --- | --- |
| 1 | Personal Protective Equipment (PPE) UAS Team 個人防護裝備 (PPE) 無人機團隊 | Checked 已檢查 | [ ] |
| 2 | Equipment Requirements (Minimum operating procedures required)EX: RC remote control、Scale 需求裝備 (最低操作程序所需)例:RC遙控器,磅秤 | Available 可用 | [ ] |
| 3 | Safety Equipment EX: Class D metal fire extinguishers 、Fire extinguishing sandbags、Emergency medical kit 安全裝備 例: D 類金屬火災滅火器 (Class D) 、滅火沙袋、緊急醫療箱 | Available, Checked 可用，已檢查 | [ ] |
| 4 | Move Aircraft from Assembly Area to RunUp Area 將飛機從組裝區移至試車區 | Min 2 People 至少2人 | [ ] |
| 5 | RunUp Area(Consider Engine Start & RunUp, 2 or more crew members are required to operate the aircraft.) 試車區(考慮引擎啟動與試車，2名或以上機組員操作) | Secure And Clear 安全且淨空 | [ ] |
| 6 | Aircraft Positioned on RunUp Area 飛機放置於試車區上 | Check Area Markings 檢查區域標記 | [ ] |
| 7 | Service Covers 機身蓋板蓋妥 | Installed, Secure 已安裝，穩固 | [ ] |
| 13 | Pitot Tube EX:4525(看飛機設計更改到360檢查) 皮托管 例:4525D | No Block And Undamaged 無堵塞損壞 | [ ] |
| 14 | Static Port EX:4525D(看飛機設計更改到360檢查) 靜壓孔 例:4525D | No Block And Undamaged 無堵塞損壞 | [ ] |
| 15 | All Covers (Pitot, Propeller, Exhaust) 所有護罩 (皮托管、螺旋槳、排氣管) | Removed 已拆除 | [ ] |
| 16 | Undercarriage 起落架 | Undamaged, Secure 無損壞，穩固 | [ ] |
| 17 | Fuselage (Lower, LHS, Upper, RHS) 機身外型 (順) | Undamaged 無損壞 | [ ] |
| 18 | All Antennas 所有天線 | Secured, Undamaged 穩固，無損壞 | [ ] |
| 21 | Runup Area、Taxiway、Runway Area/試車區、滑行道、跑道區域 | Secured And Clear安全且淨空 | [ ] |

重點提示/Important Notes:

順時針360°檢查/360°Cloclwise check(from left to right、from back to front)

| 4. Left Wing/ 左側機翼 |  |  |  |
| --- | --- | --- | --- |
| 1 | Wing (Leading Edge, Upper, Trailing Edge, Lower) 機翼 (前緣、上部、後緣、下部) (航行燈?)左紅右綠 | Undamaged 無損壞 | [ ] |
| 2 | Ailerons (Upper, Trailing Edge, Lower) 副翼 (上部、後緣、下部) | Undamaged 無損壞 | [ ] |
| 3 | Flap(Upper, Trailing Edge, Lower) 襟翼(上部、後緣、下部) | Undamaged 無損壞 |  |
| 4 | Wing Tip LED 翼尖閃光(防撞)燈 | Undamaged 無損壞 | [ ] |
| 5 | Servos 伺服馬達 | Undamaged 無損壞 | [ ] |
| 6 | Servo Covers (Covers 0.3 Nm Torque) (看飛機設計更改) 伺服馬達護蓋 (護蓋 0.3 Nm 扭力) | Secure 穩固 | [ ] |
| 7 | Servo Push Rods 伺服推桿 | Undamaged, Not Bent 無損壞，無彎曲 | [ ] |
| 8 | Servo Arms 伺服擺臂 | Undamaged 無損壞 | [ ] |
| 9 | Servo Arms 伺服擺臂 | No Play Present 無間隙 | [ ] |
| 10 | Control Surface Horns 控制面角 | Undamaged, Secure 無損壞，穩固 | [ ] |
| 11 | Pushrod Clips 推桿夾 | In Place, Secure 就位，穩固 | [ ] |
| 12 | Control Surfaces & Hinges 控制面與鉸鏈 | Operate Smoothly, No Wear or Damage 運作順暢，無磨損或損壞 | [ ] |
| 13 | 左主輪胎壓/ Left main tire pressure(燈?) | (?)PSI Normal | [ ] |
| 14 | 左主輪外觀/ Left main tire appearance | 正常無破損/Normal Undamaged | [ ] |
| 15 | 左輪擋/ Left wheel chocks | 擋妥/Installed | [ ] |

| 5. PILOT CAMERA(Nose position) / 飛行員攝影機(機頭位置) |  |  |  |
| --- | --- | --- | --- |
| 1 | Support & Cover 支架與護蓋 | Undamaged, Secure 無損壞，穩固 | [ ] |
| 2 | Camera 攝影機 | Aligned Correctly 對齊正確 | [ ] |
| 3 | Camera Lens 攝影機鏡頭 | Clean and Undamaged 乾淨無損壞 | [ ] |
| 4 | Camera Lens 攝影機鏡頭 | No Fog, Moisture or Residue 無起霧、濕氣或殘留物 | [ ] |
| 5 | 鼻輪胎壓/ Nose tire pressure | (?)PSI Normal | [ ] |
| 6 | 鼻輪外觀/ Nose tire appearance | 正常無破損/Normal Undamaged | [ ] |
| 7 | 輪擋/ wheel chocks | 擋妥/Installed | [ ] |
| 8 | 鼻輪轉向/nose wheel steering | 作動正常/Normal |  |

| 6. Right WING / 右側機翼 |  |  |  |
| --- | --- | --- | --- |
| 1 | Wing (Leading Edge, Upper, Trailing Edge, Lower) 機翼 (前緣、上部、後緣、下部) (航行燈?)左紅右綠 | Undamaged 無損壞 | [ ] |
| 2 | Ailerons (Both) (Upper, Trailing Edge, Lower) 副翼(上部、後緣、下部) | Undamaged 無損壞 | [ ] |
| 3 | Flap(Upper, Trailing Edge, Lower) 襟翼(上部、後緣、下部) | Undamaged 無損壞 |  |
| 4 | Wing Tip LED 翼尖閃光(防撞)燈 | Undamaged 無損壞 | [ ] |
| 5 | Servos 伺服馬達 | Undamaged 無損壞 | [ ] |
| 6 | Servo Covers (Covers 0.3 Nm Torque) (看飛機設計更改) 伺服馬達護蓋 (護蓋 0.3 Nm 扭力) | Secure 穩固 | [ ] |
| 7 | Servo Push Rods 伺服推桿 | Undamaged, Not Bent 無損壞，無彎曲 | [ ] |
| 8 | Servo Arms 伺服擺臂 | Undamaged 無損壞 | [ ] |
| 9 | Servo Arms 伺服擺臂 | No Play Present 無間隙 | [ ] |
| 10 | Control Surface Horns 控制面角 | Undamaged, Secure 無損壞，穩固 | [ ] |
| 11 | Pushrod Clips 推桿夾 | In Place, Secure 就位，穩固 | [ ] |
| 12 | Control Surfaces & Hinges 控制面與鉸鏈 | Operate Smoothly, No Wear or Damage 運作順暢，無磨損或損壞 | [ ] |
| 13 | 右主輪胎壓/Right main tire pressure(燈?) | (?)PSI Normal | [ ] |
| 14 | 右主輪外觀/ Right main tire appearance | 正常無破損/Normal Undamaged | [ ] |
| 15 | 右輪擋/ Right wheel chocks | 擋妥/Installed | [ ] |

| 7. Motors and propellers /馬達與螺旋槳 |  |  |  |
| --- | --- | --- | --- |
| 1 | 馬達/Motor | Rotates by Hand、Secure, Undamaged 可用手轉動、穩固，無損壞 | [ ] |
| 2 | 底座固定/ Base fixed | 螺絲鎖緊/Tighten screws | [ ] |
| 3 | 螺旋槳安裝/ Propeller installed | Undamaged, Secure 無損壞，穩固 | [ ] |
| 4 | 型號重量/Types and weight | Types: Weight: | [ ] |
| 5 | All Torque Seals(看飛機設計更改) 所有扭力標記 | Aligned 對齊 | [ ] |
| 6 | Blade 葉片 | Secure, Undamaged 穩固，無損壞 | [ ] |
| 7 | Propeller 螺旋槳 | Rotates by Hand 可用手轉動 | [ ] |
| 8 | Propeller Attachment Bolts Torque Seals (Remove Spinner to Check, Bolts 9.5 Nm Torque) 螺旋槳連接螺栓扭力標記 (拆下整流罩檢查，螺栓 9.5 Nm 扭力) (看飛機設計更改) | Aligned 對齊 | [ ] |
| 10 | Spinner 螺旋槳整流罩 | Aligned, Undamaged 對齊，無損壞 | [ ] |
| 11 | Spinner Connection Screw 整流罩連接螺絲 | Tight, No Play 鎖緊，無間隙 | [ ] |
| 12 | Standoff, Vibration Mounts 支柱，減震墊 | Undamaged, Not Hard, No Cracks 無損壞，未硬化，無裂縫 | [ ] |
| 13 | All Torque Seals 所有扭力標記(看飛機設計更改) | Aligned 對齊 | [ ] |
| 14 | 電線連接/ Wire connection | Undamaged, Not Loose, Properly Seated 無損壞，未鬆動，正確就位 | [ ] |

| 8.酬載(武掛)檢查/payload (Armored) check |  |  |  |
| --- | --- | --- | --- |
| 1 | 靜電接地（ESD）防護/ Electrostatic discharge (ESD) protection | 已連接/ Connected | [ ] |
| 2 | 檢視地面站「武器安全狀態」/ Inspect the ground station's "weapon safety status" | 安全鎖定/ DISARMED | [ ] |
| 3 | 檢查掛架（Pylon）與鎖扣結構 / Inspect the hanger (Pylon) and latching mechanism | 無鬆動、無金屬疲勞裂紋/ No loosening, no metal fatigue cracks | [ ] |
| 5 | 酬載物種類型號/payload types and models | 種類/Types: 型號/Models: | [ ] |
| 6 | 酬載物外觀/payload Appearance | Undamaged, Secure 無損壞，穩固 | [ ] |
| 7 | 酬載物重量/payload weight | (?)G/KG | [ ] |
| 8 | 減震與穩定結構檢查(防止在空中左右晃動影響飛行)/ Vibration reduction and structural stability inspection(To prevent swaying from side to side in the air from affecting flight) | 已安裝穩固/Securely And Installed | [ ] |
| 9 | 檢查保險銷/ Check the insurance pin | Installed | [ ] |

| 9. AIRCRAFT CENTRE OF GRAVITY (CG) / 飛機重心 |  |  |  |
| --- | --- | --- | --- |
| 1 | Batteries 電池 | OnBoard, Disconnected 機上，未連接 | [ ] |
| 2 | Motor馬達 | OnBoard,Installef 機上，已安裝 | [ ] |
| 3 | Ballast Required (CG Adjusted by Adding or Removing Ballast at Designated Locations) 需配重 (透過在指定位置增加或移除配重來調整重心) | Added or Removed 已增加或移除數值 | [ ] |
| 4 | Payload [ ] 酬載 [ ] | OnBoard 機上 | [ ] |
| 5 | All Protection Covers (Pitot, Propeller, Exhaust) 所有防護罩 (皮托管、螺旋槳、排氣管) | Removed 已移除 | [ ] |
| 6 | All Service Panels & Covers 所有維修面板與護蓋 | In Place 就位 | [ ] |
| 7 | Centre of Gravity 重心 | Tested & True 測試並確認無誤 | [ ] |

| 10. BATTERIES check/ 電池檢查 |  |  |  |
| --- | --- | --- | --- |
| 1 | Record Battery 1 -(EX:% Capacity & Serial Number) 記錄電池 1 -(例:電壓容量百分比與序號) | [ ] % [ ] # |  |
| 2 | Record Battery 2 -(EX:% Capacity & Serial Number) 記錄電池 2 -(例:電壓容量百分比與序號) | [ ] % [ ] # |  |
| 3 | Visual inspection/外觀檢查(無膨脹或任何型遍及裂痕) | [ ] |  |
| 4 | 確認已固定/Confirmed to be fixed | [ ] |  |
| 5 | All Cells Voltage of All Batteries 所有電池的電芯電壓 | >= 97% Capacity/容量 >= (?)V / (?)V / (?)V | normal[ ] normal[ ] |

| 11.飛行控制檢查/Fly Control Check |  |  |  |
| --- | --- | --- | --- |
| 1 | 副翼左壓桿(美手右桿)/ (Mode 2)right stick to left | 左上右下/left aileron up and right aileron down | [ ] |
| 2 | 副翼右壓桿(美手右桿)/ (Mode 2)right stick to right | 右上左下/ right aileron up and left aileron down | [ ] |
| 3 | 升降舵向前(美手右桿)/ (Mode 2)right stick foward | 前下/elevator go down | [ ] |
| 4 | 升降舵回中(美手右桿)/ (Mode 2)right stick place in the center | 中中/elevator in mid | [ ] |
| 5 | 升降舵向後(美手右桿)/ (Mode 2)rigft stick go rear | 後上/elevator go up | [ ] |
| 6 | 方向舵向左(美手左桿)/ (Mode 2)Left stick to left | 左壓左出/rudder turn left | [ ] |
| 7 | 方向舵向右(美手左桿)/ (Mode 2)Left stick to right | 右壓右出/rudder turn right | [ ] |
| 8 | 油門保持最大10sec-30sec/Keep Throttle maximum for 10sec-30sec | 穩定且無異常震動/ Stable and without abnormal vibration | [ ] |
| 9 | 各操縱系/Fly control system | 活動自如行程正確/ Activities are flexible and stick travel correct | [ ] |

------------------------------------------------ END / 結束 ------------------------------------------------